Title: ESTOP/Stop/Start of a Three-Phase Motor Circuit		Job: 18

Course: Introduction to Automation	Unit: Introduction of PLC	CLO: 4

Name ____________________________	Station _______	Date ______________

Objectives

- Student shall reinforce their knowledge of a stop/start motor control circuit.

- Student shall develop a knowledge of retentive timers and their use.

- Student shall apply this circuit in a three-phase motor control scenario.

Assessment

Students shall demonstrate a comprehension of the objectives listed above by scoring a minimum of 75% on this Job. Grading shall be based on the Introduction to PLC rubric.

Instructions

Design a stop/start motor control circuit using two momentary pushbuttons. The circuit will also utilize a latching mushroom head pushbutton to act as an “ESTOP” as well. If the ESTOP is pressed, the motor shall stop and not be able to be restarted until the “ESTOP” is disengaged. One pushbutton shall be a traditional “stop”. When pressed, it shall stop the motor. The other pushbutton shall be a traditional “start” button. When pressed the motor shall start and stay running even when the button is released. The control circuit shall also contain an “overload” function. If a motor overload occurs, the control circuit shall disable itself requiring the operator to press “start” after the overload has been reset. Whenever the motor is running, the green light shall come on and the red light shall be off. When the motor is not running, the green light shall be off and the red light shall be on. If an overload occurs, the yellow light shall illuminate. When in overload, it is permissible for both the yellow and red lights to be on simultaneously. Use the space on the opposite side of this page to design the circuit. Once complete, review the design with the instructor. After obtaining approval, configure the program in RSLogix 500. Have the instructor review the program before downloading. Once the program has been reviewed, verify and download the program.

| Input | Description | Output | Desc | Three-Phase Motor Starter |
| --- | --- | --- | --- | --- |
| I/0 | ESTOP | O/0 | Green |  |
| I/1 | 3P SS, Position A | O/1 | Yellow |  |
| I/2 | 3P SS, Position B | O/2 | Red |  |
| I/3 | 2P SS, Pos. A =ON | O/3 | Blue |  |
| I/4 | PB1, NC | O/4 | FWD Coil |  |
| I/5 | PB2, NO | O/5 | REV Coil |  |
| I/6 | PB3, NO |  |  |  |
| I/7 | Forward NO Contacts |  |  |  |
| I/8 | Overload NC Contacts |  |  |  |
| I/9 | Reverse NO Contacts |  |  |  |

Discussed design _______, Test logic without motor _______, With motor _______